# Soterix Hardware & NexaiQ

# Distributor Enablement

Contents

Hardware Portfolio	3

Purpose and Scope	3

Part Number Catalog	5

Soterix SKU Naming	7

Core Components of the SKU	7

Part Number Specification Sheets	10

CPU specifications	10

GPU specifications	10

Memory configurations	11

Storage configurations	11

Hardware Sizing Guidelines	12

Channel-based sizing philosophy	12

CPU vs GPU workload considerations	13

GPU Responsibilities	13

CPU Responsibilities	13

Storage and retention understanding	14

Additional Considerations	15

# Hardware Portfolio

## Purpose and Scope

Overview of the Soterix hardware portfolio

The Soterix hardware portfolio is a standardized, purpose-engineered family of edge platforms designed to run the NexaiQ AI video management system efficiently, predictably, and at scale. These systems are intentionally built on widely available, business-grade, and prosumer-class components rather than bespoke or proprietary hardware designs. This includes industry-standard Intel® Core™ i7 and Xeon processors, NVIDIA professional GPUs, enterprise and nearline storage devices, and a software-defined storage architecture.

This design philosophy is a deliberate engineering choice. By avoiding custom silicon, proprietary accelerators, and hardware RAID controllers, Soterix ensures that NexaiQ delivers real-time AI workloads, continuous video ingest, and long-term retention on hardware that is well understood, serviceable, and repeatable across deployments.

NexaiQ’s ability to execute AI inference, video ingest, forensic search, and retention on this class of hardware is a direct result of how its pipeline, storage model, and resource scheduling are designed. Compute, memory, and storage roles are clearly defined and tuned to the characteristics of sustained video workloads, ensuring predictable behavior under load.

All platforms in the portfolio use software-defined storage based on OpenZFS, with RAID behavior implemented at the filesystem layer rather than through dedicated hardware RAID controllers. This approach is specifically suited to video management systems, where long running ingest, delayed reads, and evidentiary integrity are critical. OpenZFS provides end-to-end data integrity through block-level check summing, explicit parity handling, and proactive maintenance operations such as scheduled scrubs. It also allows storage behavior, caching, synchronous write handling, and parity strength to be tuned to known ingest rates and retention expectations, rather than hidden behind controller firmware.

The result is a hardware portfolio that is:

- Predictable under sustained AI and video workloads

- Repeatable across deployments and environments

- Serviceable using widely available components

- Scalable through defined tiers rather than architectural redesign

- Defensible from both operational and compliance perspectives

This documentation defines the structure, intent, and boundaries of the Soterix hardware portfolio and establishes a common technical baseline for the deployment of NexaiQ at the edge.

Intended audience and usage

This documentation is intended for Soterix distributors, solution engineers, and technical sales partners responsible for specifying, positioning, provisioning, and supporting NexaiQ edge deployments. It provides clarity on why the hardware is designed as it is, how validated system performance is achieved on standard business-grade components, and which assumptions and constraints govern correct system sizing, provisioning, and deployment.

The material is designed to be used as:

- An authoritative reference for understanding Soterix hardware platforms, SKU tiers, and supported configurations

- A framework for explaining NexaiQ’s performance and efficiency on standard hardware to customers and stakeholders

- A guide for aligning customer requirements with validated system configurations and deployment models

- A reference for understanding the end-to-end workflow from certified hardware components through OS provisioning, system configuration, and NexaiQ installation

- A foundational prerequisite for advanced training on sizing, deployment, and operational support

This documentation establishes clear expectations for how Soterix-certified systems are provisioned, prepared, and brought into service, enabling the confident deployment and support of NexaiQ on validated hardware. The ability to deliver enterprise-grade AI performance through disciplined software design and standardized hardware workflows is a core differentiator of the Soterix platform.

## Part Number Catalog

The Part Number Catalog serves as the reference for identifying, ordering, quoting, provisioning, and supporting Soterix hardware platforms. It establishes a common language between Soterix, distributors, OEM partners, and customers, ensuring that every system referenced maps to a validated hardware configuration.

PVA-300 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-300-30R6/R7 | 300 | Xeon 6426Y | 3× RTX Pro 4500 | 512 GB ECC | 2× 1 TB |
| PVA-300-60R6/R7 | 300 | Xeon 6426Y | 3× RTX Pro 4500 | 512 GB ECC | 2× 1 TB |
| PVA-300-90R6/R7 | 300 | Xeon 6426Y | 3× RTX Pro 4500 | 512 GB ECC | 2× 1 TB |

PVA-200 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-200-30R6/R7 | 200 | Xeon 4510Y | 4× RTX 2000 Ada | 256 GB ECC | 2× 1 TB |
| PVA-200-60R6/R7 | 200 | Xeon 4510Y | 4× RTX 2000 Ada | 256 GB ECC | 2× 1 TB |
| PVA-200-90R6/R7 | 200 | Xeon 4510Y | 4× RTX 2000 Ada | 256 GB ECC | 2× 1 TB |

PVA-100 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-100-30NR | 100 | Core i7-14700 | 2× RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-100-60NR | 100 | Core i7-14700 | 2× RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-100-90NR | 100 | Core i7-14700 | 2× RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-100-30R5 / R6 | 100 | Core i7-14700 | 2× RTX 2000 Ada | 192 GB | 2× 1 TB |
| PVA-100-60R5 / R6 | 100 | Core i7-14700 | 2× RTX 2000 Ada | 192 GB | 2× 1 TB |
| PVA-100-90R5 / R6 | 100 | Core i7-14700 | 2× RTX 2000 Ada | 192 GB | 2× 1 TB |

PVA-75 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-75-30NR | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 1× 1 TB |
| PVA-75-60NR | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 1× 1 TB |
| PVA-75-90NR | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 1× 1 TB |
| PVA-75-30R5 / R6 | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 2× 1 TB |
| PVA-75-60R5 / R6 | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 2× 1 TB |
| PVA-75-90R5 / R6 | 75 | Core i7-14700 | RTX 4000 Ada | 128 GB | 2× 1 TB |

PVA-50 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-50-30NR | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-50-60NR | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-50-90NR | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 1× 1 TB |
| PVA-50-30R5 / R6 | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 2× 1 TB |
| PVA-50-60R5 / R6 | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 2× 1 TB |
| PVA-50-90R5 / R6 | 50 | Core i7-14700 | RTX 2000 Ada | 128 GB | 2× 1 TB |

PVA-25 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-25-30SFF | 25 | Core i7-14700 | RTX A1000 | 64 GB | 1× 1 TB |
| PVA-25-30NR | 25 | Core i7-14700 | RTX A1000 | 64 GB | 1× 1 TB |
| PVA-25-60NR | 25 | Core i7-14700 | RTX A1000 | 64 GB | 1× 1 TB |
| PVA-25-90NR | 25 | Core i7-14700 | RTX A1000 | 64 GB | 1× 1 TB |
| PVA-25-30R5 / R6 | 25 | Core i7-14700 | RTX A1000 | 64 GB | 2× 1 TB |
| PVA-25-60R5 / R6 | 25 | Core i7-14700 | RTX A1000 | 64 GB | 2× 1 TB |
| PVA-25-90R5 / R6 | 25 | Core i7-14700 | RTX A1000 | 64 GB | 2× 1 TB |

PVA-10 Edge Series

| SKU | CH | CPU | GPU | RAM | NVMe |
| --- | --- | --- | --- | --- | --- |
| PVA-10-30NR | 10 | Core i7-14700 | RTX A400 | 32 GB | 1× 1 TB |
| PVA-10-60NR | 10 | Core i7-14700 | RTX A400 | 32 GB | 1× 1 TB |
| PVA-10-90NR | 10 | Core i7-14700 | RTX A400 | 32 GB | 1× 1 TB |

### Soterix SKU Naming

Soterix SKU naming conventions establish a clear, consistent, and enforceable structure for identifying hardware platforms that are certified to run NexaiQ. The SKU name communicates the validated performance tier, deployment class, and hardware intent of the system, while ensuring that every SKU maps to a known provisioning workflow, storage model, and support boundary.

At a high level, Soterix hardware SKUs follow this structure: PVA-##-DDRL

e.g. PVA-50-30R5 is a 50-channel system, which retains 30 days of baseline video under a Raid 5 storage system.

### Core Components of the SKU

#### PVA (People, Vehicle, AI Computer Vision)

All PVA systems share the same architectural principles:

- PVA is for the standard offering, collecting and storing people and vehicle attributes and classifications through decoding and inference processes on the edge device.

- Business-grade and prosumer-class components

- Software-defined storage (ZFS Raid or NR storage)

- GPU-accelerated decode and AI inference

- Standardized OS provisioning and NexaiQ deployment workflows

No other prefixes are used for NexaiQ edge hardware.

#### ## (Channel Tier)

The channel tier (10, 25, 50, 75, 100, 200, 300) is the primary sizing and performance identifier in the SKU.

Examples: PVA-10, PVA-25, etc.

Channel tier reflects the maximum validated number of concurrent camera streams supported under the NexaiQ standard baseline:

System Sizing Per Stream Baseline

The channel tier is derived from benchmarking of GPU decode capacity, AI inference load, CPU coordination, memory behavior, and sustained ingest stability.

#### DD (Days of Retention)

DD is a two-digit retention code that states the validated number of days of stored video the system supports under the NexaiQ baseline (5MP, H.265, 12 FPS, ~3 Mbps VBR, standard GOP).

| DD Code | Meaning | What it Represents | Important Notes |
| --- | --- | --- | --- |
| 30 | 30 days retention | Validated retention tier for the SKU | If bitrate/FPS/resolution increases, the days of storage decrease |
| 60 | 60 days retention | Validated retention tier for the SKU | Retention is SKU-bound (drive count/capacity + chassis) |
| 90 | 90 days retention | Validated retention tier for the SKU | Long retention typically implies higher drive density and/or larger chassis. |

DD is a retention tier tied to the system’s storage configuration (drive count/capacity, pool layout, and chassis constraints).

#### RL (RAID Level)

RL defines the system’s storage protection model. Soterix Systems uses ZFS software-defined storage.

| RL Code | ZFS Model | Parity Level | Fault Tolerance | Intended Use |
| --- | --- | --- | --- | --- |
| NR | ZFS striped (no parity) | 0 | No parity protection | Used when the lowest cost / maximum usable capacity is prioritized, and the customer accepts a higher risk |
| R5 | ZFS RAID-Z1 | N+1 | Tolerates one drive failure | Standard parity for many production deployments (balance of protection and usable capacity) |
| R6 | ZFS RAID-Z2 | N+2 | Tolerates two drive failures | Recommended for larger pools, higher channel tiers, and longer retention (rebuild safety) |
| R7 | ZFS RAID-Z3 | N+3 | Tolerates three drive failures | Reserved for specialized/high-risk retention deployments where additional parity is required (only used when explicitly offered as a SKU tier) |

Raid Level (RL) is part of the SKU because it changes the validated storage durability profile and impacts provisioning standards (e.g.: mirrored NVMe + reserved space for SLOG on RAID systems).

Days of Retention (DD) and Raid Level (RL) are validated under the NexaiQ baseline; changes to bitrate, FPS, codec behavior, or resolution will affect achieved retention and may require a higher tier or different RL.

## Part Number Specification Sheets

### CPU specifications

|  | Cores / Threads | Base & Turbo Frequencies | Cache | Memory Support | Power |
| --- | --- | --- | --- | --- | --- |
| Intel Core i7-14700 | 20 cores (8 P-cores + 12 E-cores), 28 threads | Base: 2.1 GHz (P) / 1.5 GHz (E); Max Turbo: up to 5.4 GHz | 33 MB | Up to 192 GB DDR5-5600 or DDR4-3200; 2 channels | 65 W base; up to 219 W turbo |
| Xeon Silver 4510 | 12 cores / 24 threads | Base: 2.40 GHz; Turbo: up to 4.1 GHz | 30 MB L3 | DDR5-4800 ECC; multi-channel (server-class) | 150 W TDP |
| Xeon Gold 6426Y | 16 cores / 32 threads | Base: 2.5 GHz; Turbo: up to 4.1 GHz | 37.5 MB L3 | DDR5-4800 ECC; multi-channel (server-class) | 185 W TDP |

  - Core i7-14700 is used across PVA-10 → PVA-100 SKUs (NR, R5, R6 variants).

  - Xeon 4510Y is used exclusively in PVA-200 SKUs.

  - Xeon 6426Y is used exclusively in PVA-300 SKUs.

  - Xeon platforms are paired with ECC DDR5 and higher PCIe lane availability to support multi-GPU, rack-scale deployments.

  - CPU selection reflects coordination and orchestration needs, not AI inference (which is GPU-bound by design in NexaiQ).

### GPU specifications

| GPU Model | Architecture | CUDA Cores | Max Power (W) | NVDEC | NexaiQ Channels Supported* |
| --- | --- | --- | --- | --- | --- |
| NVIDIA RTX A400 | Ampere | 768 | 50 W | 1 | Up to 10 channels |
| NVIDIA RTX A1000 | Ampere | 2,304 | 50 W | 1 | Up to 25 channels |
| NVIDIA RTX 2000 Ada | Ada Lovelace | 2,816 | 70 W | 2 | Up to 50 channels (1× GPU) |
| NVIDIA RTX 2000 Ada (Dual) | Ada Lovelace | 5,632 (2×2,816) | 140 W | 4 | Up to 100 channels (2× GPU) |
| NVIDIA RTX 4000 Ada | Ada Lovelace | 6,144 | 130 W | 2 | Up to 75 channels (1× GPU) |
| NVIDIA RTX 2000 Ada (Quad) | Ada Lovelace | 11,264 (4×2,816) | 280 W | 8 | Up to 200 channels (4× GPU) |
| NVIDIA RTX PRO 4500 (Triple) | Blackwell | 26,880 (3×8,960) | 600 W | 6 | Up to 300 channels (3× GPU) |

  - Channel capacity values are Soterix-validated under the standard baseline (5MP, H.265, 12 FPS).

  - NVDEC count is the primary limiter for sustained ingest and decode concurrency; CUDA cores scale inference throughput but do not override decode limits.

  - Multi-GPU configurations scale linearly for decode and inference in NexaiQ.

  - GPU class defines SKU tier; GPUs are not interchangeable across SKUs without redefining the SKU.

### Memory configurations

| System | CPU Class | Total Memory | DIMM Configuration | Memory Type | ECC | Applicable SKUs |
| --- | --- | --- | --- | --- | --- | --- |
| PVA-10 | Core i7-14700 | 32 GB | 2 × 16 GB | DDR5 UDIMM | No | PVA-10-30NR / 60NR / 90NR |
| PVA-25 | Core i7-14700 | 64 GB | 2 × 32 GB | DDR5 UDIMM | No | PVA-25-NR / R5 / R6 (all retentions) |
| PVA-50 | Core i7-14700 | 128 GB | 4 × 32 GB | DDR5 UDIMM | No | PVA-50-NR / R5 / R6 |
| PVA-75 | Core i7-14700 | 128 GB | 4 × 32 GB | DDR5 UDIMM | No | PVA-75-NR / R5 / R6 |
| PVA-100 (NR / R6) | Core i7-14700 | 128 GB | 4 × 32 GB | DDR5 UDIMM | No | PVA-100-NR / R6 |
| PVA-100 (R5) | Core i7-14700 | 192 GB | 4 × 48 GB | DDR5 UDIMM | No | PVA-100-R5 |
| PVA-200 | Xeon 4510 | 256 GB | 8 × 32 GB | DDR5-5600 RDIMM | Yes | PVA-200-NR / R5 / R6 |
| PVA-300 | Xeon 6426Y | 512 GB | 8 × 64 GB | DDR5-5600 RDIMM | Yes | PVA-300-NR / R5 / R6 |

  - Memory is SKU-bound. It is not field-selectable or customer-configurable.

  - ECC is mandatory on all Xeon-based systems (PVA-200 and PVA-300).

  - Core i7 platforms use DDR5 UDIMM /Xeon platforms use DDR5 RDIMM ECC.

### Storage configurations

| HDD Capacity Class | Drive Type | Interface | Intended Use | Typical Role |
| --- | --- | --- | --- | --- |
| 14 TB | Nearline HDD | SATA 6 Gb/s | Entry & mid-tier retention | Lower-density arrays, smaller chassis |
| 18 TB | Nearline HDD | SATA 6 Gb/s | Mid-tier retention | Balanced density vs rebuild time |
| 22 TB | Nearline HDD | SATA 6 Gb/s | High-density retention | Higher channel / longer retention |
| 24 TB | Nearline HDD | SATA 6 Gb/s | Maximum density | Rack-scale and long-retention systems |

## Hardware Sizing Guidelines

### Channel-based sizing philosophy

Soterix hardware systems are sized using a channel-based methodology, where a “channel” represents a continuously ingested, decoded, analyzed, and stored video stream processed by NexaiQ. Channel count is the primary sizing dimension because it directly reflects sustained system workload rather than peak or burst activity.

All published channel capacities are validated against a standard operating baseline:

This baseline is used consistently across benchmarking, SKU definition, and support validation to ensure predictable and repeatable system behavior.

Channel-based sizing is used because NexaiQ workloads scale multiplicatively rather than linearly. Each additional channel increases demand across multiple subsystems simultaneously:

- GPU decode engines

- GPU AI inference pipelines

- CPU coordination and orchestration

- Memory utilization (including ZFS ARC)

- Continuous write-heavy storage ingest

Sizing by channel count ensures that all of these subsystems remain within validated operating envelopes under sustained load.

Channel tiers are intentionally fixed (10, 25, 50, 75, 100, 200, 300). Intermediate or fractional tiers are not supported, as they encourage oversubscription and unpredictable performance. Moving to a higher tier is the correct response when capacity is exceeded.

### CPU vs GPU workload considerations

NexaiQ systems follow a GPU-first workload model.

### GPU Responsibilities

The GPU is the primary determinant of channel capacity. It performs:

GPU class and GPU count directly determine how many channels a system can sustain—multi-GPU systems scale channel capacity by explicitly load-balancing workloads across GPUs.

### CPU Responsibilities

The CPU plays a critical but non-limiting role. It is responsible for:

CPU upgrades do not increase channel capacity once the GPU becomes a bottleneck. For this reason, Core-i7 processors are used up through PVA-100, while Xeon CPUs are introduced only when required for:

- Multi- GPU PCIe Lane availability

- ECC memory support

- Rack-scale stability and fault isolation

This separation of responsibilities ensures efficient use of hardware and prevents unnecessary cost or complexity.

Key principle:

- Increasing GPU NVDEC throughput capacity increases channel count.

- Increasing CPU capacity increases system headroom and stability.

### Storage and retention understanding

Storage sizing is driven by retention requirements, not channel capacity alone. While channel count determines ingest rate, retention determines how much data must be safely stored over time.

Soterix Systems uses OpenZFS software in all RAID hardware. Storage models are defined as:

- NR – ZFS striped (no parity)

- R5 – ZFS RAID-Z1 (single-parity) N+1

- R6 – ZFS RAID-Z2 (dual-parity) N+2

- R7 – ZFS RAID-Z3 (triple-parity) N+3

Retention tiers (30 / 60 / 90 days) are SKU-bound and correspond to validated combinations of:

- HDD capacity class (14 TB / 18 TB / 22 TB / 24 TB)

- HDD count

- Chassis size and cooling capacity

- RAID parity level

Higher channel tiers and higher retention durations require:

- Larger HDD capacities to limit drive count

- Stronger parity (RAID-Z2) to manage rebuild risk

- Larger chassis (3U / 4U) for airflow and power stability

Operating outside validated retention tiers, by extending retention or mixing drive classes, is not supported and can compromise performance, rebuild safety, and evidentiary integrity.

### Additional Considerations

- Channel count determines the system tier

- Retention determines the storage configuration within that tier

- Storage upgrades do not increase channel capacity

- Increasing retention without increasing tier increases risk

For deployments with regulatory, evidentiary, or high-availability requirements, RAID-Z2 is strongly recommended for 75 or more channels.

Soterix hardware sizing follows a clear hierarchy:

- Channels define the system tier

- GPU defines channel capacity

- CPU ensures coordination and stability

- Memory supports GPU and storage behavior

- Storage is sized for retention and durability

This approach ensures predictable performance, operational stability, and defensible system behavior across all NexaiQ deployments.