# Hardware Platform Presentation

v.1.0

Contents

Hardware Platform Presentation	3

PVA Platform Overview	3

Portfolio Overview Grid	3

Tier Details	4

PVA‑10 (10 Channels)	4

PVA‑25 (25 Channels)	4

PVA‑50 (50 Channels)	5

PVA‑75 (75 Channels)	5

PVA‑100 (100 Channels)	5

PVA‑200 (200 Channels)	6

PVA‑300 (300 Channels)	6

Storage Architecture Models	7

Configuration Selection Guidance	10

Disk Count Differences	12

GPU Differences and Computer Vision Processing Capacity	14

OS Installation Overview	16

Supported Operating Systems	16

Partitioning Standards	16

OS Partitioning	16

Data and Application Partitions	17

HDD Usage (All Systems)	17

High-Level Installation Workflows	18

Install OS	18

Run NexaiQ Installer	19

# Introduction

This document defines the Soterix PVA (Processing Video Appliance) hardware platform and its validated role as the edge compute foundation for NexaiQ deployments. It is intended to give OEMs, integrators, and internal technical teams a clear, authoritative reference for how Soterix designs, sizes, configures, and deploys edge video analytics systems across the full portfolio—from small, cost-sensitive sites to high-density, enterprise-scale environments.

The PVA platform is not a collection of generic servers. Each tier is a validated, SKU-bound system built around a specific and repeatable workload baseline: 5MP, H.265, 12 FPS, ~3 Mbps VBR per channel. Channel capacity, retention duration, RAID model, GPU configuration, memory sizing, and storage architecture are intentionally constrained to eliminate ambiguity, reduce deployment risk, and ensure predictable performance over the life of the system.

This guide explains:

- How the PVA portfolio scales by channel tier (10 through 300)

- Why GPU configuration is the primary driver of video decode and AI inference capacity

- How retention and RAID choices change the storage risk profile—and when stronger parity is required

- The standardized partitioning, mount points, and ZFS models that NexaiQ depends on operationally

- The supported operating system, installation flow, and post-OS enablement expectations

Where relevant, the document also provides design intent and rationale, not just specifications. This includes why certain RAID options are restricted at higher tiers, why NVMe space is reserved for specific roles, and why larger systems shift toward multi-GPU and stronger parity models rather than simply “scaling up” single components.

The goal is alignment: ensuring that hardware selection, deployment decisions, and field installations all match the assumptions under which NexaiQ is validated and supported. Deviations from these models are explicitly called out as unsupported or higher-risk, so expectations are clear before systems are deployed into production.

This hardware platform documentation complements Soterix’s broader installation and operating guides and should be used as the source of truth for PVA system selection and configuration

## PVA Platform Overview

The PVA (Processing Video Appliance) portfolio is Soterix’s validated edge hardware stack for NexaiQ. Each tier is defined by:

- Channel tier (10 / 25 / 50 / 75 / 100 / 200 / 300)

- DD (Days of Storage: 30 / 60 / 90)

- RL (RAID Level: NR / R5 / R6)

All tiers are validated against the NexaiQ baseline (5MP, H.265, 12 FPS, ~3 Mbps VBR).

PVA 10 / 25 / 50 / 75 / 100 / 200 / 300 SKUs

Quick Tier Summary (what changes as you move up)

- GPU class / GPU count increases → primary driver of channel capacity

- HDD required storage (TB) increases with DD and tier

- NVMe count changes by RL (NR vs RAID)

- RAM scales by tier and (in some cases) RL

### Portfolio Overview Grid

| PVA Tier | Channels | CPU | GPU Config | RAM | DD Options | RL Options | NVMe (OS/DB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PVA-10 | 10 | Core i7-14700 | RTX A400 | 32GB | 30/60/90 | NR | 1× 1TB |
| PVA-25 | 25 | Core i7-14700 | RTX A1000 | 64GB | 30/60/90 | NR | 1× 1TB (NR) / 2× 1TB (RAID) |
| PVA-50 | 50 | Core i7-14700 | RTX 2000 Ada | 128GB | 30/60/90 | NR R5 R6 | 1× 1TB (NR) / 2× 1TB (RAID) |
| PVA-75 | 75 | Core i7-14700 | RTX 4000 Ada | 128GB | 30/60/90 | NR R6 | 1× 1TB (NR) / 2× 1TB (RAID) |
| PVA-100 | 100 | Core i7-14700 | 2× RTX 2000 Ada | 192GB | 30/60/90 | NR R6 | 1× 1TB (NR) / 2× 1TB (RAID) |
| PVA-200 | 200 | Xeon 4510Y | 4× RTX 2000 Ada | 256GB | 30/60/90 | R6 | 2× 1TB (RAID) |
| PVA-300 | 300 | Xeon 6426Y | 2x RTX PRO 4500 | 512GB | 30/60/90 | R6 | 2× 1TB (RAID) |

## Tier Details

### PVA‑10 (10 Channels)

Best fit: Small sites and low camera counts with basic AI inference and retention.

- Channels: 10 (validated under the NexaiQ baseline)

- CPU: Core i7‑14700

- GPU: RTX A400 (corrected)

- RAM: 32GB DDR5 UDIMM

- ECC: No (desktop-class memory)

- NVMe (OS/DB): 1× 1TB

- DD options: 30 / 60 / 90

- RL options: NR only

- Storage: Video storage mounts at /media/vdata, application/db at /media/dbdata

### PVA‑25 (25 Channels)

Best fit: Standard small/medium deployments where RAID choice drives the durability conversation.

- Channels: 25

- CPU: Core i7‑14700

- GPU: RTX A1000

- RAM: 64GB DDR5 UDIMM

- ECC: No

- NVMe (OS/DB): 1× 1TB (NR) / 2× 1TB (RAID)

- DD options: 30 / 60 / 90

- RL options: NR / R5 / R6

  - RAID levels are implemented via ZFS software-defined storage.

- RAID systems reserve NVMe space specifically for SLOG provisioning (do not consume for general storage).

- Standard mounts: /media/dbdata and /media/vdata

### PVA‑50 (50 Channels)

Best fit: Mid-tier systems where GPU decode/inference headroom and retention requirements start to drive chassis/storage choices.

- Channels: 50 (validated under the NexaiQ baseline)

- CPU: Core i7‑14700

- GPU: RTX 2000 Ada

- RAM: 128GB DDR5 UDIMM

- ECC: No

- NVMe (OS/DB): 1× 1TB (NR) / 2× 1TB (RAID)

- DD options: 30 / 60 / 90

- RL options: NR / R6

- RAID 6.

- Standard mounts: /media/dbdata and /media/vdata

### PVA‑75 (75 Channels)

Best fit: High Computer Vision Processing throughput in a single-GPU tier; common step-up when 50-channel systems are tight on decode/inference.

- Channels: 75 (validated under the NexaiQ baseline)

- CPU: Core i7‑14700

- GPU: RTX 4000 Ada

- RAM: 128GB DDR5 UDIMM

- ECC: No

- DD options: 30 / 60 / 90

- RL options: NR / R6

- NVMe (OS/DB): 1× 1TB (NR) / 2× 1TB (RAID)

- RAID 6.

- Standard mounts: /media/dbdata and /media/vdata

### PVA‑100 (100 Channels)

Best fit: High channel density and concurrency. Dual-GPU tier for decode/inference scaling.

- Channels: 100 (validated under the NexaiQ baseline)

- CPU: Core i7‑14700

- GPU: 2× RTX 2000 Ada

- RAM: 128GB (NR) / 192GB (R6) DDR5 UDIMM

- ECC: No

- NVMe (OS/DB): 1× 1TB (NR) / 2× 1TB (RAID)

- DD options: 30 / 60 / 90

- RL options: NR / R6

- RAID 6.

- Standard mounts: /media/dbdata and /media/vdata

### PVA‑200 (200 Channels)

Best fit: Enterprise channel density requiring server-class platform stability (ECC, PCIe lanes) and multi-GPU scaling.

- Channels: 200 (validated under the NexaiQ baseline)

- CPU: Xeon 4510

- GPU: 4× RTX 2000 Ada

- RAM: 256GB DDR5‑5600 RDIMM ECC

- NVMe (OS/DB): 2× 1TB (RAID)

- ECC: Yes (required on Xeon tiers)

- DD options: 30 / 60 / 90

- RL options: R6 only

- Mirrored OS/DB NVMe with 1TB NVME for SLOG.

- Standard mounts: /media/dbdata and /media/vdata

### PVA‑300 (300 Channels)

Best fit: Highest-density tier for large deployments that require maximum concurrency and sustained throughput.

- Channels: 300 (validated under the NexaiQ baseline)

- CPU: Xeon 6426Y

- GPU: 2× RTX PRO 4500

- RAM: 512GB DDR5‑5600 RDIMM ECC

- NVMe (OS/DB): 2× 1TB (RAID)

- ECC: Yes (required on Xeon tiers)

- DD options: 30 / 60 / 90

- RL options: R6 only

- Mirrored OS/DB NVMe with 1TB NVME for SLOG.

- Standard mounts: /media/dbdata and /media/vdata

## Storage Architecture Models

All models below are ZFS software-defined and are built to support sustained NexaiQ video ingest workloads.

| Model | SKU Code | ZFS Layout | What it protects | What it costs |
| --- | --- | --- | --- | --- |
| Non-RAID | NR | Striped / no parity | No disk-fault protection | Highest usable capacity; lowest durability |
| Z1 | R5 | RAID-Z1 (single parity) | Survives 1 disk failure in a vdev | ~1 disk of parity per vdev |
| Z2 | R6 | RAID-Z2 (dual parity) | Survives 2 disk failures in a vdev | ~2 disks of parity per vdev |
| Z3 | R7 | RAID-Z3 (triple parity) | Survives 3 disk failures in a vdev | ~3 disks of parity per vdev |

Non-RAID configurations (NR)

NR is offered for cases where the customer prioritizes maximum usable capacity and simplicity, and where the operational plan accepts higher risk (for example, shorter retention, lower channel counts, or an external backup/replication strategy).

NR is also operationally simple because it avoids parity write overhead and resilver planning; however, it provides no disk-fault tolerance, and any HDD failure in the ingest pool affects stored video evidence.

RAID configurations (Z1 / Z2 / Z3)

RAID options exist because video retention systems are write-heavy, long-running, and evidence-oriented, and the risk profile changes rapidly as pools grow and rebuild times increase.

Soterix’s guidance explicitly recognizes that as channel tier and retention increase, the storage configuration must shift toward stronger parity to control rebuild risk: higher channel tiers and longer retention durations require stronger parity (RAID Z2) to manage rebuild risk.

How to think about Z1 vs Z2/Z3

- Z1 (R5 / RAID-Z1) is a capacity-optimized parity model. It is appropriate only when the customer accepts the higher risk profile inherent to single parity rebuild windows. A single error during array reconstruction could compromise the entire array.

- Z2 (R6 / RAID-Z2) is the default durability model for larger pools because it materially improves survivability during rebuild/resilver events.

- Z3 (R7 / RAID-Z3) is reserved for the highest-risk retention scenarios (very large pools, very high retention, or strict evidentiary/compliance requirements).

Why Z2/Z3 became the only RAID options past 50 channels / 30 days

Once you cross ~50 channels and/or >30 days of retention, the storage pool size and disk counts typically move into a range where single parity rebuild windows become a meaningful operational risk, and even at 50 channels / 30 days, that risk can still exist depending on drive size and vdev width.

That’s why, if RAID is requested, Soterix’s portfolio direction is:

- Z2 (R6) is the default parity model for sustained production workloads at higher tiers.

- Z3 (R7) is used when the risk profile demands it (largest pools / strict evidence requirements).

This aligns with the enablement guidance that stronger parity is required as channel tiers and retention durations grow, and it explicitly recommends RAID‑Z2 for higher-reliability deployments at scale.

Why PVA-200 and PVA-300 is critical to run at Z2 or Z3

At 200 and 300 channels, you are operating at rack-scale ingest rates and disk counts. In that regime:

- A rebuild/resilver event is not a rare edge case; it’s an operational reality over the life of the system.

- The cost of a second failure during a rebuild window becomes unacceptable for evidence workloads.

So, for these tiers, Soterix standardizes on:

- Z2 (R6) minimum, with Z3 (R7) as the escalation option when the durability requirement is extreme.

Two Z2 pools under /media/vdata” guidance for 200/300 tiers

For 200/300-channel tiers, there’s an additional durability and operations tactic that fits the Soterix mountpoint standard:

- Keep the video ingest storage mounted at /media/vdata (required for consistency across NexaiQ nodes and scripts).

- Build the ingest storage as two independent Z2 fault domains under that mountpoint (commonly implemented as two RAID‑Z2 vdev groups inside one pool, rather than a single very wide vdev).

This reduces “blast radius,” shortens resilver exposure per fault domain, and improves parallelism for sustained ingest and replay.

This approach keeps the operational contract intact (same mount point) while reducing risk in the highest-density deployments.

For RAIDZ ingest nodes, Soterix’s storage standard also ties RAID models to NVMe log behavior:

- The workload baseline used for sizing and expectations is H.265, 5MP, 12 FPS, ~3 Mbps VBR per channel.

- NVMe SLOG is used to improve synchronous write acknowledgment latency (ingest stability) where required; it does not increase sustained HDD throughput.

## Configuration Selection Guidance

Storage model selection for NexaiQ edge systems should be treated as a durability + retention decision (not a “performance upgrade”), because channel tier sets the compute envelope. In contrast, retention sets the storage envelope within that tier. Increasing retention or changing drive classes outside validated combinations is not supported and increases rebuild and evidentiary data corruption risk.

Considerations When Choosing a Storage Model

| Storage Model | What it means in Soterix terms | Best-fit use cases |  | Key cautions / why you’d move up |
| --- | --- | --- | --- | --- |
| NR (Non-RAID) | Single-copy video storage (no parity protection). | Cost-sensitive deployments where the customer accepts that a single disk failure can cause video loss; shorter retention needs; “good enough” evidence requirements. | Most appropriate at lower tiers / lower retention where loss tolerance is explicitly acceptable. | As retention and disk count rise, rebuild window + failure exposure increases, so NR becomes harder to justify in any evidentiary or availability-driven deployment. |
| Z1 (R5 / RAID-Z1) | Single-parity ZFS RAIDZ (1 drive worth of parity). The SKU label “R5” denotes a RAID 5 class offering. | Small-to-mid arrays where the customer needs some fault tolerance but is still cost-constrained. | This is the “entry RAID” option, use only when the customer understands the tradeoff (single parity). | At higher channel tiers and/or longer retention, single parity is not enough to manage rebuild risk; this is why we steer RAID requests to Z2/Z3 as systems scale. |
| Z2 (R6 / RAID-Z2) | Dual-parity ZFS RAIDZ (N+2). | Default choice for regulatory/evidence, higher availability expectations, and larger arrays. | Strongly recommended once you reach higher channel tiers and real-world evidence expectations. RAID Z2 is strongly recommended for 75 channels or more. | Dual parity is used to manage rebuild risk as retention duration and HDD capacity class increase. This is the “safe default” RAID posture. |
| Z3 (R7 / RAID-Z3) | Triple-parity ZFS RAIDZ (N+3). | Rack-scale / longest-retention / highest-risk exposure deployments where resiliency is non-negotiable. | Use when the customer’s retention + scale profile makes rebuild exposure unacceptable under dual parity alone. | Z3 is about failure-tolerance margins and defensibility under prolonged rebuild windows (big nearline drives + sustained ingest). |

Why we offer different retention tiers

- Retention tiers (30/60/90 days) are SKU-bound and are validated combinations of HDD capacity class, drive count, chassis/cooling, and RAID parity level.

- As you move up in channels and/or retention, the storage system necessarily shifts toward larger nearline drives and longer rebuild windows, which increases exposure during a failure event, so the portfolio moves toward stronger parity (Z2, then Z3) to keep deployments stable and defensible.

- This is also why the guidance warns that increasing retention without increasing tier increases risk (because you’re pushing the storage envelope harder without changing the validated platform envelope).

The “50 channels / 30 days” inflection

- At and beyond mid-tier deployments, the combination of continuous ingest + high-density HDDs + retention makes rebuild exposure a real operational and retention risk.

- Practically, that means: if a customer requests RAID above ~50 channels and/or beyond 30 days, the configuration change to Z2 (or Z3, where required) is the natural progression for evidentiary retention levels. Z2 and Z3 are aligned to mitigate rebuild risk.

PVA-200 and PVA-300 guidance

- For large tiers, the storage stance should be Z2 or Z3 only in practice (even if someone asks for “RAID” without specification), because the rebuild/fault exposure at that scale is not aligned with evidence-grade outcomes.

- With much larger disk arrays, a single ZFS pool mounted at /media/vdata can be built from multiple RAIDZ2 vdev groups (i.e., multiple protected groupings inside one pool). Hence, the system has the right durability envelope under sustained ingest. The standard requires the pool to be mounted consistently at /media/vdata.

## Disk Count Differences

HDD Sizing Approach

HDD sizing is based on usable capacity, not advertised capacity. Drive manufacturers rate disks in decimal terabytes, while the actual usable space is lower after formatting and is accounted for by the filesystem. From that usable capacity, Soterix operates each drive at no more than 80% utilization, maintaining headroom for sustained ingest, metadata growth, and stable ZFS behavior.

When calculating storage requirements, this 80% utilization target is applied consistently across all disks. If the calculated requirement approaches or exceeds that threshold, the next higher drive capacity is selected. For example, if three 10 TB drives are marginally insufficient at the 80% utilization target, the configuration moves to three 12 TB drives rather than increasing disk count or operating closer to full capacity. This approach prioritizes predictability, rebuild safety, and long-term stability over maximizing theoretical capacity.

Unique HDD capacity classes used for sizing (and what they really hold)

These are the unique HDD TB classes used in the sizing logic, with the corresponding usable capacity assumptions from the same HDD Logic tab:

| Advertised (TB) | Usable (TiB) | 80% Usable (TiB) |
| --- | --- | --- |
| 14 | 12.73 | 10.18 |
| 18 | 16.37 | 13.1 |
| 22 | 20.01 | 16.01 |
| 24 | 21.83 | 17.46 |

Capacity implications

- Drive size directly controls drive count.
Example deltas from the table below:

  - PVA-100-90: 28×14TB vs 16×24TB (12 fewer drives)

  - PVA-200-90: 55×14TB vs 32×24TB (23 fewer drives)

  - PVA-300-90: 82×14TB vs 48×24TB (34 fewer drives)

- More drives = more chassis and operational load.

  - Higher drive counts increase: rack space needs, power draw, cooling demand, HBA/backplane requirements, and overall failure surface area.

- RAID adds parity overhead on top of capacity planning.

  - If the SKU is RAID-based (R5/R6/R7), the design must also account for:

    - minimum vdev width for the chosen RAIDZ level, and

    - parity capacity overhead (which is exactly why high-tier systems move to Z2/Z3 when RAID is required).

- Why 200/300 tiers push Z2/Z3 (and often multiple protected groups):

  - Even with 24TB drives, the raw HDD count at 60–90 days becomes large enough that parity strategy and fault-domain design (ex: multiple protected groups under /media/vdata) becomes a durability requirement.

Drive quantity by SKU

PVA-##-DDRL represents the tier and retention days (DD). The RL suffix (NR / R5 / R6 / R7) changes the parity model, but the video TB requirement is the same for a given tier + DD.

| SKU_key | Video TB @ baseline | 14TB HDDs | 18TB HDDs | 22TB HDDs | 24TB HDDs |
| --- | --- | --- | --- | --- | --- |
| PVA-10-30 | 9.70 | 1 | 1 | 1 | 1 |
| PVA-10-60 | 18.54 | 2 | 2 | 2 | 2 |
| PVA-10-90 | 27.81 | 3 | 3 | 2 | 2 |
| PVA-25-30 | 23.17 | 3 | 2 | 2 | 2 |
| PVA-25-60 | 46.35 | 5 | 4 | 3 | 3 |
| PVA-25-90 | 69.52 | 7 | 6 | 5 | 4 |
| PVA-50-30 | 46.35 | 5 | 4 | 3 | 3 |
| PVA-50-60 | 92.70 | 10 | 8 | 6 | 6 |
| PVA-50-90 | 139.05 | 14 | 11 | 9 | 8 |
| PVA-75-30 | 69.52 | 7 | 6 | 5 | 4 |
| PVA-75-60 | 139.05 | 14 | 11 | 9 | 8 |
| PVA-75-90 | 208.57 | 21 | 16 | 14 | 12 |
| PVA-100-30 | 92.70 | 10 | 8 | 6 | 6 |
| PVA-100-60 | 185.39 | 19 | 15 | 12 | 11 |
| PVA-100-90 | 278.09 | 28 | 22 | 18 | 16 |
| PVA-200-30 | 185.39 | 19 | 15 | 12 | 11 |
| PVA-200-60 | 370.79 | 37 | 29 | 24 | 22 |
| PVA-200-90 | 556.18 | 55 | 43 | 35 | 32 |
| PVA-300-30 | 278.09 | 28 | 22 | 18 | 16 |
| PVA-300-60 | 556.18 | 55 | 43 | 35 | 32 |
| PVA-300-90 | 834.27 | 82 | 64 | 53 | 48 |

## GPU Differences and Computer Vision Processing Capacity

NexaiQ follows a GPU-first workload model:

- GPU = decode + inference + parallel stream processing (this is what drives channel tier)

- CPU = coordination/orchestration, DB/indexing, ZFS/system services (important, but not the channel limiter once GPU is saturated)

Two key implications for distributors:

- NVDEC count is the primary limiter for sustained decode concurrency

- CUDA cores scale inference throughput but cannot “override” decode limits.

- Multi-GPU scales linearly for decode + inference.

GPU models per SKU tier

| PVA Tier | CH | GPU Configuration | Why that GPU config exists |
| --- | --- | --- | --- |
| PVA-10 | 10 | 1x RTX A400 | Entry tier for small deployments; validated baseline assumptions apply |
| PVA-25 | 25 | 1x RTX A1000 | 1× NVDEC-class decode + inference capacity sized to the 25ch baseline envelope |
| PVA-50 | 50 | 1x RTX 2000 Ada | Higher NVDEC + CUDA headroom to sustain 50ch decode + AI workloads concurrently |
| PVA-75 | 75 | 1x RTX 4000 Ada | Higher-tier single-GPU envelope for 75ch sustained AI processing throughput |
| PVA-100 | 100 | 2× RTX 2000 Ada | Multi-GPU scaling for decode + inference; partitioned workload sustains 100ch |
| PVA-200 | 200 | 4× RTX 2000 Ada | Four dual-slot GPUs with higher performance, lower thermals, power draw, and cost over the RTX 4000 Ada’s |
| PVA-300 | 300 | 3× RTX PRO 4500 | Triple-GPU envelope for sustained 300ch decode + AI at baseline |

Sizing Guidance

Unless high-density deployment is an absolute requirement, Soterix recommends meeting higher channel counts by deploying multiple edge systems rather than concentrating all load on a single appliance.

Distributing capacity reduces thermal density, power draw, rebuild exposure, and single-point-of-failure risk, while avoiding the operational complexity associated with very large nearline disk arrays.

What changes the AI workload?

Compute load isn’t linear—it’s the combined effect of the stream profile:

If one parameter increases, the compute cost increases; if multiple parameters increase, the cost compounds.

The practical levers that change “AI Inferencing throughput.”

- Resolution: The higher the frame resolution, the more work the edge appliance has to do to process the frame. 8MP is ~60% more work per inference cycle than 5MP → fewer channels per system.

- FPS: Higher FPS drives more decode operations and more frames entering inference. At 20–30 FPS, inference load can double or triple.

- Codec: AI can’t analyze compressed frames—frames must be decoded first. H.265 is smaller on the wire but heavier to decode and consumes more GPU decode resources per stream, reducing channel headroom faster.

- Bitrate/VBR: Even at the same resolution, higher bitrate increases detail/noise/texture → more decode + more AI evaluation work. A 5MP stream at 5–6 Mb/s behaves materially differently than 3 Mb/s for compute load.

- GOP (I-frame interval): Since NexaiQ inference is on I-frames, GOP controls inference frequency. Typical GOP=10 at 12 FPS ≈ one inference-worthy frame every ~0.83 seconds; shorter GOP (e.g., >1:1) causes a massive increase in inference + decode load.

# OS Installation Overview

## Supported Operating Systems

Soterix NexaiQ edge systems are certified and supported on the following operating systems:

- Ubuntu 22.04 LTS Server (Jammy Jellyfish)

All provisioning scripts, drivers, container runtimes, and NexaiQ services are validated against this version.

## Partitioning Standards

All systems follow a fixed, SKU-driven partitioning model to ensure predictable behavior across OS services, databases, and video storage workloads. Partition layouts differ between Non-RAID (NR) and RAID-based (Z1–Z3) systems.

OS, DB, and Video Storage Partitioning is performed during initial OS installation and must not be altered post-deployment. ZFS RAID Systems will provision HDDs after the OS is installed. ZFS HDDs are unformatted and unmounted during OS install.

### OS Partitioning

Non-RAID (NR) Systems

NR systems use a single 512GB NVMe device for OS and application data.

| Device | Mount Point | Size | Purpose |
| --- | --- | --- | --- |
| NVMe (512GB) | / | 220GB | Operating system and core services |
| 1x 512GB Non-RAID | /media/dbdata | 250GB | NexaiQ databases, metadata, and application data |
|  | Unallocated | ~5GB | Reserved (no use) |

- /media/dbdata isolates the database and application I/O from the OS.

- No SLOG is used in NR systems.

RAID-Based Systems (Z1 / Z2 / Z3)

RAID systems use two 1TB NVMe devices in a mirrored (RAID-1) configuration for OS and application data.

| Device | Mount Point | Size | Purpose |
| --- | --- | --- | --- |
| NVMe (1TB) | / | 400GB | Operating system and core services |
| 2x 1TB RAID-1 | /media/dbdata | 450GB | NexaiQ databases, metadata, and application data |
|  | SLOG | ~80GB | Reserved exclusively for SLOG provisioning |

- OS and database partitions are mirrored for resilience.

- The remaining NVMe space is reserved and must not be formatted.

- This space is later carved for ZFS SLOG per channel tier and RAID requirements.

- No other use of the reserved NVMe space is permitted.

### Data and Application Partitions

NVMe Usage Summary

| System Type | NVMe Configuration | Purpose |
| --- | --- | --- |
| NR | Single 512GB NVMe | OS + application data |
| Z1–Z3 | 2× 1TB NVMe (RAID-1) | OS, application data, SLOG |

### HDD Usage (All Systems)

All system HDDs, regardless of RAID model, are dedicated exclusively to video storage.

| Device Type | Mount Point | Purpose |
| --- | --- | --- |
| All HDDs | /media/vdata | Video ingest and retention (ZFS datasets) |

- /media/vdata is the only valid mount point for video storage.

- ZFS pools are created on HDDs only.

- No OS, database, or application data is stored on HDDs.

- Partition layouts are SKU-bound and must not be modified.

- /media/dbdata is mandatory on all systems.

- /media/vdata is reserved exclusively for video storage.

- NVMe space reserved for SLOG must remain unformatted until ZFS provisioning.

- Mixing roles across partitions is not supported.

# High-Level Installation Workflows

### Install OS

### Run NexaiQ Installer