# NexaiQ

# ZFS RAID REQUIREMENTS

Contents

Introduction	3

ZFS RAID Requirements	4

SLOG (Separate Log Device) Requirements	5

SLOG Requirements	6

NVMe Endurance Class Requirements (TBW / DWPD)	7

ARC: Purpose and Rules	8

ARC Configuration Requirements	8

ARC sizing policy (by SKU RAM class and channel tier)	8

ARC Target Values for Standard SKUs	9

ARC Behavior Expectations	9

ARC and Dataset Interaction	10

ARC Verification Requirements	10

# Introduction

This document defines the authoritative ZFS RAID storage standard for NexaiQ edge video-ingest edge platforms. Its purpose is to establish a repeatable, auditable, and benchmark-validated baseline for how ZFS must be configured, tuned, and operated in production, independent of specific HDD models, vendors, or chassis form factors. The focus is on behavioral correctness, data durability, and operational consistency, not on component selection.

The requirements herein are derived from empirical benchmarking, long-duration ingest testing, and field-oriented analysis across RAID-Z1 and RAID-Z2 configurations. They reflect the realities of sustained video ingest workloads, specifically H.265, 5MP, 12 FPS, variable bitrate streams at scale, where storage systems must balance throughput, latency, integrity, and recoverability without relying on unsafe shortcuts or undocumented assumptions

This standard intentionally separates requirements, rationale, and verification:

- Requirements state what must be configured.

- Rationale explains why each setting exists, grounded in ZFS mechanics and observed performance.

- Verification provides concrete commands so installers, QA, support, and auditors can independently confirm compliance.

Key design principles embedded in this document include:

- Parity-first durability using RAIDZ with proper sector alignment.

- Dataset-level tuning that distinguishes large sequential video ingest from small, metadata-heavy metrics workloads.

- Explicit ARC governance to prevent memory contention with analytics and application services.

- Mandatory data-integrity maintenance via scheduled scrubs and SMART testing.

- Formalized synchronous write behavior for video evidence workloads, ensuring durability semantics are preserved.

- Controlled use of NVMe SLOG devices, scaled by channel tier, to reduce synchronous write acknowledgment latency while maintaining crash safety.

This document is intended to be normative. Deviation from these requirements, particularly around sync, logbias, SLOG usage, or scrub discipline, must be deliberate, documented, and approved, as such changes directly affect data integrity and evidentiary reliability. When followed as written, this standard provides a storage foundation that is predictable under load, resilient under failure, and defensible in both operational and compliance contexts.

# ZFS RAID Requirements

| Requirement | Rationale | Verification |
| --- | --- | --- |
| Pool must be ZFS RAIDZ (Z1/Z2/Z3) using ashift=12 | Ensures parity protection and 4K sector alignment for HDD-based workloads. | zpool status shows RAIDZ vdevs; zdb -C confirms ashift=12 |
| Pool name must be 'zdata' and mounted at /media/vdata | Provides consistent mountpoints across all NexaIQ nodes and scripts. | zfs list ¦ grep zdata shows mountpoint /media/vdata |
| All disks referenced via /dev/disk/by-id | Prevents device renaming issues across reboots or hardware changes. | zpool status shows by-id paths only |
| Dataset zdata/video_ingest with recordsize=1M, compression=lz4, atime=off | Optimized for large sequential video writes with minimal metadata overhead. | zfs get recordsize, compression,atime zdata/video_ingest |
| Dataset zdata/metrics with recordsize=128K, compression=lz4, atime=off | Optimized for small-file writes and frequent metadata access. | zfs get recordsize, compression,atime zdata/metrics |
| ARC max/min must be explicitly set | Prevents ZFS from starving application memory on high-RAM systems. | cat /sys/module/zfs/parameters/zfs_arc_max |
| Monthly ZFS scrub enabled via systemd | Detects and repairs silent corruption proactively. | systemctl status zfs-scrub-monthly@zdata.timer |
| SMART monitoring enabled with daily short and weekly long tests | Provides early detection of disk failure trends. | smartctl -a /dev/disk/by-id/* and smartd logs |

# SLOG (Separate Log Device) Requirements

Workload baseline for sizing: H.265, 5MP, 12 FPS, 3 Mbps VBR per channel.

Ingest payload rate (approx): 3 Mbps/channel = 0.375 MB/s/channel (3 ÷ 8). SLOG improves synchronous write acknowledgment latency (ZIL) when logbias=latency and sync is not disabled; it does not increase the sustained throughput of the HDD RAID vdevs.

| Channel Tier | Payload Ingest (Mbps) | Payload Ingest (MB/s) | Conservative ZIL Working Set (2 min) | Required SLOG NVMe Partition |
| --- | --- | --- | --- | --- |
| 25 | 75 | 9.375 | 1.1 GB | 200 GB (256GB) |
| 50 | 150 | 18.750 | 2.2 GB | 300 GB (512GB) |
| 75 | 225 | 28.125 | 3.3 GB | 400 GB (512GB) |
| 100 | 300 | 37.500 | 4.4 GB | 500 GB (512GB) |
| 200 | 600 | 75.000 | 8.8 GB | 800 GB (1TB) |
| 300 | 900 | 112.500 | 13.2 GB | 1000 GB (1TB) |

Notes:

SLOG is required on all RAIDZ HDD video-ingest nodes where the ingest dataset uses logbias=latency and sync is not disabled.

The sizing above standardizes partitions per SKU tier using a 1TB NVMe. The 2-minute ZIL working set shows actual log space needs are small; the larger partitions are for standardization and endurance margin.

Do not share the SLOG partition with OS/application partitions. Prefer a dedicated NVMe device; if partitioned, ensure the SLOG partition is isolated and uses by-id paths.

## SLOG Requirements

| Requirement | Rationale | Verification |
| --- | --- | --- |
| Video ingest pool MUST include an NVMe SLOG (log vdev) when sync is enabled (sync=standard) and logbias=latency. | Benchmarks show SLOG materially reduces synchronous write acknowledgment latency (ZIL), improving ingest stability under sync-heavy NVR behavior, even though it does not raise HDD aggregate throughput. | zpool status shows a LOG vdev; zfs get sync, logbias on ingest dataset |
| SLOG device MUST be NVMe with power-loss protection (PLP) and high endurance; SLOG MUST NOT be a consumer DRAM-less NVMe device. | ZIL/SLOG is in the write-ack path for synchronous transactions; PLP prevents data loss/corruption on sudden power loss, and high endurance supports continuous writes. | Confirm NVMe model/spec (vendor datasheet); smartctl -a /dev/nvmeX shows media wear; confirm PLP |
| SLOG partition sizing on 1TB NVMe MUST follow tier standard: 25ch=200GB; 50ch=300GB; 75ch=400GB; 100ch=500GB; 200ch=800GB; 300ch=1000GB. | Standardizes builds while providing a generous endurance margin. Payload ingest rates scale linearly with channel count, at 3 Mbps per channel. | lsblk shows partition size; zpool status shows log vdev uses that partition |
| SLOG must be referenced via /dev/disk/by-id and remain ONLINE. | Prevents device renaming issues and ensures ZIL durability. | zpool status shows by-id path and ONLINE state for LOG |
| Video ingest dataset (zdata/video_ingest) MUST use sync=standard and logbias=latency in production; sync=disabled is prohibited for field deployments. | This formalizes the synchronous write behavior expected for NVR ingest and ensures the SLOG provides value (via the ZIL ack path). Disabling sync bypasses ZIL/SLOG and trades durability for speed, which is not acceptable for production video evidence workloads. | zfs get sync, logbias zdata/video_ingest (expect sync=standard, logbias=latency) |

## NVMe Endurance Class Requirements (TBW / DWPD)

Because the SLOG (ZIL log device) absorbs high-frequency synchronous write traffic, the NVMe used for SLOG must be selected and validated not only for power-loss protection (PLP), but also for write endurance. Endurance must be expressed as either DWPD or TBW (manufacturer rating). Drives that meet PLP but fall into low-end consumer endurance classes are not acceptable for continuous 24×7 ingest workloads.

Minimum endurance class

- SLOG NVMe devices must be rated at ≥ 1.0 DWPD (or the equivalent TBW rating for the drive capacity) across the warranty period.

- If DWPD is not published, use TBW and verify it meets or exceeds the equivalent of 1.0 DWPD × warranty years.

How endurance is validated
Endurance compliance is validated using a combination of:

- Manufacturer published DWPD/TBW rating (datasheet), and

- Field monitoring of NVMe wear indicators (SMART / NVMe log pages), including:

  - Percentage used / media wear

  - Data units written

  - Host writes and write amplification indicators (if exposed)

This ensures the SLOG device remains reliable over the expected service life of the system and prevents premature wear-out under sustained synchronous traffic.

Requirement
SLOG NVMe MUST meet PLP requirements and minimum endurance class of ≥ 1.0 DWPD (or equivalent TBW for capacity and warranty term).

Rationale
SLOG devices receive continuous write cycles under synchronous workloads; insufficient endurance increases failure risk and maintenance burden even when PLP is present.

Verification

- Confirm DWPD/TBW from vendor datasheet

- Validate in-field using:

  - smartctl -a /dev/nvmeX (percentage used, data units written, media wear)

  - Confirm device remains within expected wear rate during burn-in and 24-hour ingest simulation

# ARC: Purpose and Rules

The ZFS Adaptive Replacement Cache (ARC) is the primary in-memory cache for ZFS data and metadata. In NexaIQ edge systems, ARC is a performance helper that must remain strictly bounded so it accelerates filesystem operations without competing with the RAM required for video ingest, analytics, containers, databases, and the OS.

ARC provides value primarily through:

- Faster metadata traversal (file create/open/close, directory lookups, inode activity)

- Reduced HDD seek amplification during mixed ingest + read activity

- Smoother operation during bursty contention (short spikes in reads, indexing, metrics writes)

NOTE: Because ARC aggressively consumes free RAM when uncapped, explicit ARC limits are mandatory for all production builds.

## ARC Configuration Requirements

ARC limits MUST be explicitly set

ARC sizing must not rely on defaults. Each SKU class must define both ARC maximum and ARC minimum using a persistent module configuration.

Required configuration file

/etc/modprobe.d/zfs_arc.conf

## ARC sizing policy (by SKU RAM class and channel tier)

Observed steady-state “Required RAM to run” (application + OS) scales by channel count and is modeled using the upper-bound curve:

RequiredRAM_max(ch) = 35GB + (0.2GB × channels)

Operational headroom is reserved per RAM class to protect burst capacity and prevent memory pressure:

- 64GB systems: ReserveHeadroom = 12GB

- 128GB systems: ReserveHeadroom = 20GB

- 256GB systems: ReserveHeadroom = 32GB

- 512GB systems: ReserveHeadroom = 64GB

- ARC maximum is then calculated using two guards:

  - Do not encroach on required RAM + headroom

- ARCmax_candidate = TotalRAM − RequiredRAM_max(ch) − ReserveHeadroom

  - Prevent excessive ARC growth on large-RAM systems

- ARCmax_cap = 60% × TotalRAM

Final ARC maximum

- ARCmax = min(ARCmax_candidate, ARCmax_cap)

This policy ensures ARC remains predictable, provides meaningful cache benefit, and never crowds out the runtime memory NexaIQ needs.

## ARC Target Values for Standard SKUs

| Tier | RAM | Required RAM_max | Reserve Headroom | ARCmax_ candidate | 60% Cap | ARCmax (set) | ARCmin (set) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 64GB | 40GB | 12GB | 12GB | 38GB | 12GB | 4GB |
| 75 | 128GB | 50GB | 20GB | 58GB | 76GB | 58GB | 16GB |
| 100 | 128GB | 55GB | 20GB | 53GB | 76GB | 53GB | 16GB |
| 200 | 256GB | 75GB | 32GB | 149GB | 154GB | 149GB | 16GB |
| 300 | 512GB | 95GB | 64GB | 353GB | 307GB | 307GB | 16GB |

## ARC Behavior Expectations

ARC is expected to function primarily as:

- A metadata accelerator for ingest file creation, directory operations, and metrics/log activity

- A short-term read cache, not a long-term store

ARC is not intended to:

- Mask undersized HDD pools

- Replace correct dataset recordsize tuning

- Compensate for missing SLOG behavior or incorrect sync/logbias policy

ARC supports stability indirectly by improving filesystem efficiency; ingest durability and write correctness are governed by dataset properties and pool design.

## ARC and Dataset Interaction

Dataset cache directives influence ARC effectiveness:

- primarycache=all on zdata/video_ingest allows caching of metadata and selective hot reads.

- primarycache=metadata on zdata/metrics prioritizes metadata acceleration without filling ARC with low-value data blocks.

Any changes to dataset cache properties must be treated as performance-sensitive and validated under sustained ingest load.

## ARC Verification Requirements

After modifying ARC parameters, the system must be rebooted to enforce the limits at runtime.

Verify configured values

cat /sys/module/zfs/parameters/zfs_arc_max

cat /sys/module/zfs/parameters/zfs_arc_min

Values must match the target ARCmax/ARCmin for the system’s channel tier and installed RAM class.

Optional operational checks during load testing:

- ARC stabilizes below zfs_arc_max

- No sustained memory pressure or swap escalation is introduced